DAM-124 IP SET

①Connect the mixer and your computer to your router. Follow the steps in the image to set the fixed IP address of the mixer.

Open configuration software

1：Select the network card that connects your computer to your router.

2：Click Search. (If no connected device is displayed after the search, check whether the connection is correct or whether the network adapter is selected correctly.)

3：In the device bar, double-click the device you want to set.

②As shown below

1：Deselect the On function in DHCP.

2：Set a fixed IP address in the IP address bar (according to the customer's LAN Settings, the picture is the IP address used for testing)

3：Click SET ALL to send the parameters to the device for setting and saving.

4：After the device is set successfully and restarts automatically, the system displays that the restart is complete.

③After the Settings are complete, read the parameters again to check whether the Settings are successful.

1：Click Search.

2：Check whether the IP address of the device is set successfully.

Note: If you need to set to automatically assign IP addresses. Select the DHCP On function again and click SET ALL. The restart is complete.